# Project Phase-I: SmartScan – Intelligent Document Enhancement and Segmentation System

### Group Information

- E235 – Parth Chaturvedi

- E236 – Divyansh Karel

- E250 – Himanshu Garg

### 1. Introduction & Problem Statement

Document images captured via smartphone cameras frequently suffer from non-uniform ambient illumination, sharp shadows, low optical contrast, sensor noise, and complex background clutter. These physical degradations compromise visual legibility and hinder the performance of downstream text recognition and archival pipelines. SmartScan is a classical digital image processing system designed to restore, enhance, and segment degraded document images using deterministic image processing algorithms.

### 2. Project Objectives

- Eliminate shadow gradients and normalize illumination using spatial and point-processing enhancement techniques.

- Extract document boundaries and text regions via gradient-based edge detection and adaptive thresholding.

- Refine binarized text strokes and eliminate background noise artifacts using morphological operations.

- Benchmark system performance quantitatively using Peak Signal-to-Noise Ratio (PSNR) and visual comparison plots.

### 3. System Methodology & Pipeline

The SmartScan framework executes through a modular, four-stage image processing sequence:

- Preprocessing & Spatial Enhancement: Input RGB document images are converted to grayscale. 2D Median Filtering is applied to eliminate impulsive scanner and sensor noise. Contrast-Limited Adaptive Histogram Equalization (CLAHE) and power-law transformations adjust global and local dynamic contrast across uneven shadow regions.

- Boundary Extraction & Feature Analysis: Gradient operators (Sobel) and optimal multi-stage detectors (Canny) identify physical document boundaries and structural line features.

- Adaptive Document Segmentation: Global Otsu thresholding and Local Adaptive Binarization are implemented to partition foreground text strokes from non-uniform background surfaces.

- Morphological Post-Processing: Structuring elements execute morphological Opening (to remove non-text isolated noise specks) and Closing/Dilation (to bridge broken stroke discontinuities in damaged characters).

### 4. Work Distribution

| Member | Assigned Role | Core Responsibilities |
| --- | --- | --- |
| Parth Chaturvedi (E235) | Image Preprocessing & Enhancement | Implementation of 2D median filters, linear/piecewise contrast stretching, and CLAHE. |
| Divyansh Karel (E236) | Edge Detection & Segmentation | Implementation of Sobel/Canny edge detectors, Global Otsu, and Local Adaptive Binarization. |
| Himanshu Garg (E250) | Morphology & System Integration | Implementation of morphological opening/closing operations, pipeline integration, and PSNR evaluation. |

### 5. Syllabus Mapping & Development Environment

- Syllabus Units Covered: Unit 2 (Image Enhancement), Unit 4 (Morphological Processing), Unit 5 (Edge Detection), and Unit 6 (Image Segmentation).

- Course Outcomes Mapped: CO1 (Image Fundamentals), CO2 (Enhancement & Filtering), CO3 (Edge Detection & Morphology), and CO4 (Segmentation).

- Development Environment: MATLAB R2023b / R2024a (Image Processing Toolbox).

- Expected Output: A modular MATLAB script / UI displaying multi-stage intermediate and final processed images alongside quantitative quality metrics.